C64 Expansion Port Extension Cartridge Side Rev. 0

Module Description

# Introduction

The purpose of the C64 Expansion Port Extension is to place an Expansion Port connector at a desired location and connect the actual C64 Expansion Port via a ribbon cable. The maximum length of said ribbon cable is not yet determined it is >25cm and <100 cm.  This board is providing the cartridge connector and a reset switch (two possible positions).

Figure 1: Dimension of the cartridge side PCB

# Connector Pinouts

## J2 – Break out connector

2x25 box header, 2.54mm pitch

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | GND | 2 | GND |
| 3 | +5V | 4 | GND |
| 5 | +5V | 6 |  |
| 7 | +5V | 8 |  |
| 9 |  | 10 |  |
| 11 | R / | 12 | PHI2 |
| 13 | DOT CLOCK | 14 | A15 |
| 15 |  | 16 | A14 |
| 17 |  | 18 | A13 |
| 19 |  | 20 | A12 |
| 21 |  | 22 | A11 |
| 23 |  | 24 | A10 |
| 25 | BA | 26 | A9 |
| 27 |  | 28 | A8 |
| 29 | D7 | 30 | A7 |
| 31 | D6 | 32 | A6 |
| 33 | D5 | 34 | A5 |
| 35 | D4 | 36 | A4 |
| 37 | D3 | 38 | A3 |
| 39 | D2 | 40 | A2 |
| 41 | D1 | 42 | A1 |
| 43 | D0 | 44 | A0 |
| 45 | GND | 46 | GND |
| 47 | GND | 48 | GND |
| 49 | GND | 50 | GND |

## J3 – Cartridge Connector

Card Edge Connector 22x2P 2.54mm (0.1“) Right Angle

| Pin (TOP) | Signal | Pin | Signal (BOT) |
| --- | --- | --- | --- |
| 1 | GND | A | GND |
| 2 | +5V | B |  |
| 3 | +5V | C |  |
| 4 |  | D |  |
| 5 | R / | E | PHI2 |
| 6 | DOT CLOCK | F | A15 |
| 7 |  | H | A14 |
| 8 |  | J | A13 |
| 9 |  | K | A12 |
| 10 |  | L | A11 |
| 11 |  | M | A10 |
| 12 | BA | N | A9 |
| 13 |  | P | A8 |
| 14 | D7 | R | A7 |
| 15 | D6 | S | A6 |
| 16 | D5 | T | A5 |
| 17 | D4 | U | A4 |
| 18 | D3 | V | A3 |
| 19 | D2 | W | A2 |
| 20 | D1 | X | A1 |
| 21 | D0 | Y | A0 |
| 22 | GND | Z | GND |

# Revision History

## Rev. 0

The prototype is functional with a 25cm ribbon cable.

See test description (Doc.-No. 227-6-02-00)